Shri G. S. Institute of Technology & Science, Indore

Department of Computer Engineering

Subject: Computer Interfacing & IoT [CO44003]

Session:  July – December 2020

Lab Assignment

Submitted To:						                Submitted By:

Mr. Veerendra Shrivastava					      Shikhar Mahajan

Assistant Professor					                0801CS171077

Index

| Sr. No. | Assignment | Page No. | Submission Date | Signature |
| --- | --- | --- | --- | --- |
| 1. | Lab Assignment 1 - features of EdSim51 - Addressing modes - Assembly language program - Copy 5 byte data block - Sum of first 10 natural numbers | 2 2 3 5 14 15 | 15-12-2020 |  |

Lab Assignment 1

Q1. Download and install the EdSim51 simulator for 8051. Study and write the basic features of EdSim51.

Answer:

- The bottom panel shows all the peripherals that are connected to the 8051.

- Syntax Highlighting : assembly code written in EdSim51 is automatically syntax highlighted

- Originally, the simulator ran with a system clock frequency of 12 MHz. Now the user can enter a value for the system clock frequency in MHz.

- A list of 8-bit numbers (written in HEX) can be transmitted. To do so, the user encloses the list in curly braces, each number separated by a comma.

- A simulation of the popular Hitachi HD44780 LCD module has been implemented for the EdSim51 Simulator. And now CGRAM has also been implemented.

- For high resolution monitors, click on the zoom button. The zoom button is located below the red Exit button.

- A breakpoint can be set by double-clicking the instruction's address.

- We can now save our source code in Intel HEX format and also we can write C code for the 8051 using one of many available 8051 C compilers, then import the HEX code into the EdSim51 Simulator.

- When running a program, the rate at which the screen updates is determined by the setting in the Update Freq. menu

- Load and Save - The user  can write code directly into the text box when it is in edit mode, or an existing program can be loaded from a file using the Load button. Similarly, code in the text box can be saved to file using the Save button

- Keypad Modes - User can select from three modes of operation-standard,pulse ,radio

Q2. Study the 8051 addressing modes. Write and execute/simulate the assembly language program in EdSim51 based on following addressing modes:

Answer:

Addressing modes are an aspect of the instruction set architecture in most central processing unit designs. The various addressing modes that are defined in a given instruction set architecture define how the machine language instructions in that architecture identify the operand of each instruction.

- Immediate Addressing Mode: In this mode data is present in the address field of instruction .

MOV A,#05h

- Register Addressing Mode: In register addressing the operand is placed in one of 8 bit or 16 bit general purpose registers.

MOV A,#08h

MOV R0,A

- Direct Addressing Mode

mov 80H,#05H // location 80H has data 5

MOV A,80H   // 5 moved to Accumulator

- Indirect Addressing Mode

Location 80H has data 3, whose address is saved in Ro, whose value is moved to A

MOV 80H,#03H

MOV R0,80H

MOV A,R0

Q3. Write and execute/simulate the assembly language program in EdSim51 based on:

Answer:

- Arithmetic Operations

- Addition

MOV A,#2h

MOV R0,#3h

ADD A,R0

- Subtraction

MOV A,#6h

MOV R0,#3h

SUBB A,R0

- Multiplication

MOV A,#2h

MOV B,#3h

MUL AB

MOV R0,A

- Division

MOV A,#8h

MOV B,#2h

MUL AB

DIV R0,A

- Conditional Instruction

- JZ

Initialize A with 5 unit after 3 unit of counter

MOV R0,#3h

loop:

MOV A,R0

DEC R0

JZ loop1

JNZ loop

loop1:

MOV A,#05h

- JNZ

Timer/Counter for 3 unit

MOV R0,#3h

loop:

MOV A,R0

DEC R0

JNZ loop

- JNC

MOV A, #02H

MOV A, #04H

JNC L1

L1: MOV A,#03H

- Loop Instructions

- CJNE - Counter of 5 unit

MOV R0,#0h

loop:

INC R0

CJNE R0,#5h,loop

- DJNZ-Counter of 8 unit

MOV R0,#8h

loop:

DJNZ R0,loop

- Logical Instructions

- NOT:

MOV A, #5BH

CPL A

- AND:

MOV A,#1101B

MOV R0,#1011B

ANL A,R0

- XOR:

MOV A,#1100B

MOV R0,#1010B

XRL A,R0

- OR:

MOV A,#1100B

MOV R0,#1010B

ORL A,R0

- Shifting Operations

- Rotate Left:

MOV A, #1010B

RL A

- Rotate Right:

MOV A, #1010B

RR A

- Rotate Right with Carry:

MOV A, #1101B

RRC A

- Rotate Left with Carry:

MOV A, #1101B

RLC A

Q4. Write and execute/simulate the assembly language program in EdSim51 to copy a 5Byte data block from one memory location to another memory location.

Start:

MOV R0, #30H

MOV A, #00H

MOV R7, #07H

MOV R2, #0200H

BACK :  MOV A, R0

MOV R2, A

INC R0

INC R2

DJNZ R7, BACK

END

Q5. Write and execute/simulate the assembly language program in EdSim51 to sum of first 10 natural numbers. Store the sum in some register.

Answer:

ORG 0000h

MOV R0,#0Ah

MOV R1,#01h

loop:

ADD A,R1

INC R1

DJNZ R0, loop

MOV R2,A

End